GM Oil ATF Specifications

1. History

Dexron Type A, Suffix A

Specification introduced in 1957. It requires the oil to meet certain limits regarding its kinematic viscosity.

Dexron IID

General Motors Dexron®-IID Specification. ATF issued in 1975. Contained ATF cooler corrosion requirements not listed in Dexron® - II.

Dexron IIE

General Motors Specification Dexron®-IIE. ATF issued in 1991 requiring improved low temperature performance compared to Dexron®-IID, 20 000 cP at minus 40 °C.

Dexron IIIF

GM specification for Automatic transmission oil introduced in 1994. Successor of Dexron IID and IIE.

Dexron IIIG

Successor of Dexron III(F) automatic transmission fluid. This has the same low temperature characteristics as Dexron IIE, but with modifications to anti-oxidancy and friction material. Introduced in 1997.

Dexron IIIH

Dexron III licence H was introduced in June 2003 to replace the Dexron III G fluid. It has an oxidatively stable base oil (group 2 or group 3). Oils according to this specification have longer maintenance of friction properties and anti-shrudder properties, better foam control and a longer fluid life.

Dexron VI

Specification introduced in 2005 to replace Dexron IIIH. This specification requires better stay-in-grade properties, oxidative stability and anti-foam characteristics. Oils meeting this specification can be used with extended drain intervals and are energy conserving.

* Then back number(dexron XX) is higher, get more improved friction stability, viscosity stability, oxidation resistance and so on.

2. Contents

|  | GT ATF Type-II | GT ATF Type-III | GT ATF Type-IV Multi Vehicle (High viscosity) | ATF MLV 6 (Low viscosity) |
| --- | --- | --- | --- | --- |
| Spec | Dexron IID- obsolete | Dexron III(H)- obsolete | Dexron VI- continued and so on | Dexron VI- continued and so on |
| Kinematic Viscosity, CSt, @100℃ | 6.xx | 6.xx | 7.xx | 5.xx |
| feature | 4-5 speed | 4-5 speed With Lock Up Clutch | 4-6 speed Multi spec | Only 6 speed Low friction & viscosity |